FE to PE Release Checklist

This checklist ensures all Fabrication Engineering (FE) deliverables are compliant with company standards before handoff to Production Engineering (PE). Complete these steps to ensure a high-quality handoff and minimize fabrication delays.

By marking ‘Released’ in Epicor and completing the FE -> PE Release Form in the dedicated channel, you certify that your deliverables are conformant to all of the following requirements:

- Approval Prerequisite

- Confirm that the job has been fully approved by all necessary parties.

  - Build Strategy: Project Advisor has reviewed and approved. “PA Approved” is checked in Epicor

  - Client Review: Client has reviewed submittal (drawings, models, mockups, samples) and approved. “Submittal Status” is set to Approved in Epicor. “Submitted Date” gives the date of last submittal / resubmittal.

1. Rhino Model Geometry & Hygiene

- Run Cleanup: Execute Purge to remove unused blocks/layers/styles and SelDup to delete duplicate geometry.

  - No layouts and corresponding layers / blocks present.

  - No empty layers present.

  - No nested “assembly” blocks present.

- Object Integrity: Confirm no "Bad Objects" (SelBadObjects), no "Open Polysurfaces" (SelOpenPolysrf), and no "Extrusions" (SelExtrusion).

  - Fix: Use ConvertExtrusion to change extrusions to polysurfaces.

- Part Name Uniqueness Audit: : Ensure no unlike geometries share the same part name.

  - Step-Through Audit: Use the SelName command to select objects by name. Step through the list to verify that each unique name refers only to identical parts (or a single part).

  - Naming Conflicts: If unlike geometries share a name, differentiate them by selecting the next unused P### number in series for the shipping component (2x P005 becomes P005 and P011).

- Naming Conventions: Verify all names match standard conventions for database compatibility.

  - File Name (Release): ####.###_PE_YYYYMMDD.3dm (e.g., 1234.567_PE_20240512.3dm).

  - Polysurfaces: ####.###.SC###.P### (e.g., 1234.567.SC001.P001 or for MFG parts: 1234.MT.00567.P002).

  - Block Definitions:

    - Hardware (Embedded): Must start with a bracketed hardware tag (e.g., <GM.HW.12345> Description).

    - Manufactured (Linked): Must follow the MFG parts format (e.g., <1234.MT.00567> Description ).

  - Note: All names that include a description must align with corresponding Epicor description field.

  - Note: Catalog items may begin with a 2 character account code rather than project number.

- Metadata (UserText): Ensure all standard Document and Object UserText keys (thickness, volume, project ID, etc) are populated via the Engineering Toolkit.

- Material Thickness: Verify that modeled thicknesses match physical materials within company standard tolerances. Contact Production Manager to confirm stock-on-hand thicknesses for each project.

  - Commonly used materials should have thicknesses recorded in Epicor part attributes for reference.

    - Non-finish grade plywood (“shop ply”) is typically undersized relative to nominal thickness. FE should model undersized in accordance with historical trends. (e.g model 0.75” nominal shop ply at 0.72”)

    - Dimensionally stable materials (MDO, MDF) and custom sheet goods (Pre-lam veneered layups, etc) are typically within expected tolerance, and should be modeled at nominal thickness.

  - Any uncommon or specialty sheet good is required to be checked with a micrometer.

- Bent Metal: Modeled with correct inside bend radius and material thickness. Verify that 2D flat patterns/unfolds are provided with appropriate bend deductions.

2. Layering & Organization

- Layer Names:

  - Must begin with the <GM.&&.#####> material tag to correspond with the BOM (e.g., 03_CWKA-FE::PUR::SG::<GM.SG.12345> Description).

  - All handoff layers reside under the 03_CWKA-FE::PUR or 03_CWKA-FE::MFG parent.

    - If a project warrants strict MFG part version control, “_BLOCK” files will instead use 04_CWKA-PE::PUR

- Reference Data: Resides on 03_CWKA-FE::CON child layers.

  - Construction geometry (reference surfaces, differencing cylinders for holes, guide curves, etc) must be purged.

  - “By Others” geometry may remain in the release file for reference if it dictates specific machining coordination.

3. ERP, BOM & Material Validation

- BOM Synchronization: Cross-reference all material tags (<>) in the Rhino file against the Epicor BOM.

  - All materials used in the model must appear on the Epicor BOM. If a needed material is not currently in Epicor:

    - Global materials (GM) are requested in the Part Requests channel. Availability for BOM subject to 24hr server sync cycle

    - Project-specific materials and Manufactured parts are created directly by FE in Epicor “Part Entry” dashboard, then added to BOM

  - Every material on the BOM must derive from an entry in the Epicor Parts master table, cannot be a  '“part-on-the-fly”

  - Every material must be assigned to the correct “Rel Opr.” (Related Operation)

  - Note: Any changes to the BOM made post-release must be flagged with the “Added Mtl” checkbox.

- Takeoffs: BOM gross quantity for the whole job for each material to cover the rough stock needed to produce the final geometry as modeled

  - Board Footage: Calculations completed per company standard and reflected in the Epicor requested parts.

    - Note: Solid Lumber (SL) board dimensions (min. length, min. width) must be added to the material description on the BOM.

  - Verify "Fixed QTY" is checked for each material (except CMG) so that the material will not be multiplied by Job’s parent quantity.

  - Reminder that Engineering should not add bulk overage % beyond the takeoff amount to material quantities to the BOM.

4. Production Readiness & Tooling

- Shipping Components: Model must be broken into shipping units.

  - Reminder: A shipping component is a completed rigid assembly that leaves the shop with a single identifying tag

    - Multiples of LIKE geometries can be shipped under a single Shipping Component number.

    - Loose UNLIKE geometries cannot be shipped under a single Shipping Component number.

  - ERP Dashboard Upload: Upload the Shipping Components List to Epicor’s UD40 (or UD37 for CMG) dashboard.

  - Requirement: Ensure both Component Names and Quantities are accurately reflected in the dashboard.

- Tooling Audit:

  - Check shop-specific bed sizes (e.g., NY CNC 5x12, Metal Laser 5x10).

  - Model must conform to manufacturing site-specific drill diameters

  - For Laser: Ensure hole diameters are not smaller than material thickness.

  - Support Project Mgmt / Purchasing to order any required new dies or custom tooling ahead of the Job Start Date.

- Nesting & Tolerances: All Parts must fit within sheet tolerances

  - Note: The full nominal area of the sheet is not available for nesting.

    - 1/2" perimeter clear zone for wood CNC,

    - Greater of 1/4" or material thickness perimeter clear zone for metal laser.

- Manufactured Part Jobs:

  - Note: Manufactured part must be collaboratively identified with Project Advisor (PA)

  - Submit the Job Request Form for all necessary MFG jobs and create the required folders.

    - MFG parts are released independently in advance of the parent jobs that they feed.

5. Final Handoff & Documentation

- PE Export: Acceptable filenames (prefix before _PE):

    - Standard: PROJ.NUM — e.g. 1105.007_PE.3dm, 1105.007_PE_20250124.3dm

    - Standard manufactured part: PROJ.MT or PROJ.WC + 5-digit part number — e.g. 1103.MT.00004_PE.3dm, 1105.WC.00100_PE.3dm

    - Catalog item: 2-char account + 2-char category + 3-digit number — e.g. CM.BE.001_PE.3dm

    - Catalog manufactured part: 2-char account + MT or WC + 5-digit part number — e.g. CM.MT.00005_PE.3dm, TT.WC.00100_PE.3dm

    - Isodate of release follows _PE (PROJ.JOB _PE_20250124.3dm).

  - File Organization:

    - Move the purged Rhino release file to the 03_PE_Releases/PROJ.JOB/Production_Files folder.

    - Copy the full PDF fabrication set (including Finish/Material schedules) to 03_PE_Releases/PROJ.JOB/Production_Drawings.

  - Geometry State: Models should show geometry as it comes "off the machine" (e.g., sharp corners for bench-applied round-overs).

  - PDF Standards: Ensure drawings are exported as Vector PDFs (where possible) and Shipping Components are individually noted.

- Epicor Admin (Job Level): Ensure the following fields are set:

  - "Engineering Complete”: Check the "Engineering Complete" box for every material on the BOM to confirm you have double-checked quantities and part numbers.

  - "BOM Complete", and "Field Dimensions Received" tasks are verified complete; checkboxes are checked.

  - "Fabrication Engineer" field is filled with your name. Note: Field is labeled “Production Team” on Job Entry.

- Communication & Notification:

  - Submit the FE-to-PE Form. Tag the assigned Project Manager in the resulting channel post.

    - Releases that occur after their scheduled start dates, require extra coordination with adjacent departments.

    - Please acknowledge that this coordination is underway in the notes field of the Release Post

Handoff Meeting Policy

A formal handoff meeting is mandatory if:

- The project includes custom tooling/dies.

- There are oversized parts requiring special nesting.

- The assembly involves complex non-standard "By Others" integration.

- The assembly includes details that deviate from established company norms.

Note: Use the "Notes" section of the handoff form to request a meeting with PE. PE may also request a handoff meeting upon file review.